CCNA Security

Chapter 10 - Configure ASA Basic Settings and Firewall using ASDM (Instructor Version)

(ASA-5506 / Equiv)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

|  | This lab has been updated for use on NETLAB+. www.netdevgroup.com |
| --- | --- |

Topology

Note: ISR G1 devices use FastEthernet interfaces instead of GigabitEthernet interfaces.

IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 209.165.200.225 | 255.255.255.248 | N/A | ASA G1/1 |
|  | S0/0/0 (DCE) | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
|  | S0/0/1 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 172.16.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| ASA | VLAN 1 G1/2 | 192.168.1.1 | 255.255.255.0 | NA | S2 F0/24 |
|  | G1/1 | 209.165.200.226 | 255.255.255.248 | NA | R1 G0/0 |
|  | VLAN 3 G1/3 | 192.168.2.1 | 255.255.255.0 | NA | S1 F0/24 |
| PC-A | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S1 F0/6 |
| PC-B | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S2 F0/18 |
| PC-C | NIC | 172.16.3.3 | 255.255.255.0 | 172.16.3.1 | S3 F0/18 |

Objectives

Part 1: Configure Basic Device Settings

Configure basic settings for routers and switches.

Configure static routing, including default routes, between R1, R2, and R3.

Enable the HTTP server on R1 and set the enable and VTY passwords.

Configure PC host IP settings.

Verify connectivity.

Part 2: Access the ASA Console and ASDM

Access the ASA console and view hardware, software, and configuration settings.

Clear previous ASA configuration settings.

Bypass Setup mode and configure the ASDM VLAN interfaces.

Configure ASDM and verify access to the ASA.

Access ASDM and explore the GUI.

Part 3: Configure ASA Settings and Firewall Using the ASDM Startup Wizard

Access the Configuration menu and launch the Startup wizard.

Configure the hostname, domain name, and enable the password.

Configure the inside and outside VLAN interfaces.

Configure DHCP, address translation, and administrative access.

Review the summary and deliver the commands to the ASA.

Test access to an external website from PC-B.

Test access to an external website using the ASDM Packet Tracer utility.

Part 4: Configure ASA Settings from the ASDM Configuration Menu

Set the ASA date and time.

Configure a static default route for the ASA.

Configure AAA user authentication using the local ASA database.

Test SSH access to the ASA.

Test connectivity using ASDM Ping and Traceroute.

Modify the MPF application inspection policy.

Part 5: Configure DMZ, Static NAT, and ACLs

Configure the ASA DMZ VLAN 3 interface.

Configure the DMZ server and static NAT.

View the DMZ Access Rule generated by ASDM.

Test access to the DMZ server from the outside network.

Background/Scenario

The Cisco Adaptive Security Appliance (ASA) is an advanced network security device that integrates a stateful firewall, a VPN, and other capabilities. This lab employs an ASA 5506 to create a firewall and protect an internal corporate network from external intruders while allowing internal hosts access to the Internet. The ASA creates three security interfaces: Outside, Inside, and DMZ. It provides outside users with limited access to the DMZ and no access to internal resources. Inside users can access the DMZ and outside resources.

The focus of this lab is the configuration of the ASA as a basic firewall. Other devices will receive minimal configuration to support the ASA portion of the lab. This lab uses the ASA GUI interface ASDM to configure basic device and security settings.

In Part 1 of this lab, you will configure the topology and non-ASA devices. In Part 2, you will prepare the ASA for Adaptive Security Device Manager (ASDM) access. In Part 3, you will use the ASDM Startup wizard to configure basic ASA settings and the firewall between the inside and outside networks. In Part 4, you will configure additional settings via the ASDM configuration menu. In Part 5, you will configure a DMZ on the ASA and provide access to a server in the DMZ.

Your company has one location connected to an ISP. R1 represents a customer-premise equipment (CPE) device managed by the ISP. R2 represents an intermediate Internet router. R3 connects an administrator from a network management company, who has been hired to remotely manage your network. The ASA is an edge security device that connects the internal corporate network and DMZ to the ISP while providing NAT and DHCP services to inside hosts. The ASA will be configured for management by an administrator on the internal network and the remote administrator. Layer 3 VLAN interfaces provide access to the three areas created in the lab: Inside, Outside, and DMZ. The ISP has assigned the public IP address space of 209.165.200.224/29, which will be used for address translation on the ASA.

Note: The router commands and output in this lab are from a Cisco 1941 router with Cisco IOS Release 15.4(3)M2 (with a Security Technology Package license). Other routers and Cisco IOS versions can be used. See the Router Interface Summary Table at the end of the lab to determine which interface identifiers to use based on the equipment in the lab. Depending on the router model and Cisco IOS version, the commands available and the output produced might vary from what is shown in this lab.

The ASA used with this lab is a Cisco model 5506 with an 8-port integrated router, running OS version 9.8(1), Adaptive Security Device Manager (ASDM) version 7.8(1), and comes with a Base license.

Instructor Note: Instructions for erasing switches and routers are provided in Chapter 0.0.0.0. Instructions for erasing the ASA, accessing the console, and accessing ASDM are provided in this lab.

Instructor Notes:

This lab is divided into five parts. Part 1 and 2 can be performed separately but must be performed before Parts 3 through 5. Part 2 uses the ASA CLI to prepare the ASA for ASDM Access. Parts 3 through 5 can be performed individually, or in combination with others as time permits, but they should be performed sequentially. In some cases, a task assumes the configuration of certain features in a prior task.

The main goal is to use an ASA to implement firewall and other services that might previously have been configured on an ISR. In the Chapter 9 Lab, the student configured the most common basic ASA settings and services, such as NAT, ACL, DHCP, AAA, and SSH from the CLI. In this lab, the student uses ASDM to configure these features.

The final running configs for all devices are found at the end of the lab.

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the routers, such as interface IP addresses and static routing.

Note: Do not configure ASA settings at this time.

Configure basic settings for routers and switches.

Configure hostnames, as shown in the topology, for each router.

Configure router interface IP addresses, as shown in the IP Addressing table.

Configure a clock rate for routers with a DCE serial cable attached to the serial interface. R1 is shown here as an example.

R1(config)# interface S0/0/0

R1(config-if)# clock rate 128000

Configure the hostname for the switches. With the exception of the hostname, the switches can be left in their default configuration state. Configuring the VLAN management IP address for the switches is optional.

Configure static routing on the routers.

Configure a static default route from R1 to R2 and from R3 to R2.

R1(config)# ip route 0.0.0.0 0.0.0.0 10.1.1.2

R3(config)# ip route 0.0.0.0 0.0.0.0 10.2.2.2

Configure a static route from R2 to the R1 Fa0/0 subnet (connected to ASA interface E0/0) and a static route from R2 to the R3 LAN.

R2(config)# ip route 209.165.200.224 255.255.255.248 10.1.1.1

R2(config)# ip route 172.16.3.0 255.255.255.0 10.2.2.1

Configure and encrypt passwords on R1.

Note: Passwords in this task are set to a minimum of 10 characters and are relatively simple for the purposes of performing the lab. More complex passwords are recommended in a production network.

      - Configure a minimum password length. Use the security passwords command to set a minimum password length of 10 characters.

R1(config)# security passwords min-length 10

      - Configure the enable secret password on both routers with a password of cisco12345. Use the type 9 (SCRYPT) hashing algorithm.

R1(config)# enable algorithm-type scrypt secret cisco12345

      - Create a local admin01 account using admin01pass for the password. Use the type 9 (SCRYPT) hashing algorithm and set privilege level to 15

R1(config)# username admin01 privilege 15 algorithm-type scrypt secret admin01pass

Configure the Console and VTY lines to use the local database for login. For additional security, configure the lines to log out after five minutes of inactivity. Issue the logging synchronous command to prevent console messages from interrupting command entry.

R1(config)# line console 0

R1(config-line)# login local

R1(config-line)# exec-timeout 5 0

R1(config-line)# logging synchronous

R1(config)# line vty 0 4

R1(config-line)# login local

R1(config-line)# exec-timeout 5 0

R1(config-line)# logging synchronous

R1(config-line)# transport input ssh

Enable HTTP server access on R1. Use the local database for HTTP authentication.

R1(config)# ip http server

R1(config)# ip http authentication local

Note: HTTP server access will be used to demonstrate ASDM tools in Part 3.

Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A, PC-B, and PC-C as shown in the IP Addressing table.

Verify connectivity.

There will be no connectivity between devices that are connected to the ASA because the ASA is the focal point for the network zones and it has not been configured. However, PC-C should be able to ping the R1 interface G0/0. From PC-C, ping the R1 G0/0 IP address (209.165.200.225). If these pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Note: If you can ping from PC-C to R1 G0/0 and S0/0/0, you have demonstrated that addressing has been configured properly, and static routing is configured and functioning correctly.

Save the basic running configuration for each router and switch.

Access the ASA Console and ASDM

In Part 2, you will access the ASA via the console and use various show commands to determine hardware, software, and configuration settings. You will prepare the ASA for ASDM access and explore ASDM screens and options.

Access the ASA console.

If prompted to enter Interactive Firewall configuration (Setup mode), answer No.

Enter privileged mode with the enable command and password (if set). The password is blank by default, so press Enter. If the password has been changed to one that is specific to this lab, enter the password cisco12345. The default ASA hostname and prompt is ciscoasa>.

ciscoasa> enable

Password: cisco12345 (or press Enter if no password is set)

Clear previous ASA configuration settings.

Use the write erase command to remove the startup-config file from flash memory.

ciscoasa# write erase

Erase configuration in flash memory? [confirm]

[OK]

ciscoasa#

ciscoasa# show start

No Configuration

Note: The erase startup-config IOS command is not supported on the ASA.

Use the reload command to restart the ASA. This causes the ASA to come up in CLI Setup mode.
If you see the message: “System config has been modified. Save? [Y]es/[N]o:” Type N and then press Enter.

CCNAS-ASA# reload

Proceed with reload? [confirm]

CCNAS-ASA#

***

*** --- START GRACEFUL SHUTDOWN ---

Shutting down isakmp

Shutting down webvpn

Shutting down sw-module

Shutting down License Controller

Shutting down File system

*** --- SHUTDOWN NOW ---

Process shutdown finished

Bypass Setup mode and configure the ASDM VLAN interfaces.

When the ASA completes the reload process, it should detect that the startup-config file is missing and present a series of interactive prompts to configure basic ASA settings. If it does not come up in this mode, repeat Step 2.

When prompted to pre-configure the firewall through interactive prompts (Setup mode), respond with no.

Pre-configure Firewall now through interactive prompts [yes]? no

Enter privileged EXEC mode with the enable command. The password should be blank (no password) at this point.

Enter global configuration mode using the conf t command. The first time you enter configuration mode after reloading, you will be prompted to enable anonymous reporting. Respond with no.

Configure the inside interface Gi1/2 to prepare for ASDM access. The Security Level should be automatically set to the highest level of 100. The Gi1/2l interface will be used by PC-B to access ASDM on the ASA.

ciscoasa(config)# interface gi1/2

ciscoasa(config-if)# nameif inside

INFO: Security level for "inside" set to 100 by default.

ciscoasa(config-if)# ip address 192.168.1.1 255.255.255.0

ciscoasa(config-if)# security-level 100

ciscoasa(config-if)# exit

PC-B is connected to switch S2. Switch S2 is connected to ASA port Gi1/2. Why is it unnecessary to add physical interface Gi1/2 to a VLAN?

____________________________________________________________________________________

____________________________________________________________________________________

Gi1/2 is a Layer 3 interface and has an IP address. It is operating as the default gateway for the 192.168.1.0 /24 subnet.

ASA 5506 interface notes:

The 5506 is different from the 5505 series ASA model. Like a Cisco router, the 5506 ASA physical ports can be directly assigned a Layer 3 IP address. The ASA 5505 has eight integrated switch ports that are Layer 2 ports.

By default, all ASA physical interfaces are administratively down unless the Setup utility has been run, or the factory defaults have been reset. Use the show interface ip brief command to verify this.

ciscoasa# show interface ip brief

Interface                  IP-Address      OK? Method Status                Protocol

Virtual0                   127.1.0.1       YES unset  up                    up

GigabitEthernet1/1         unassigned      YES unset  administratively down down

GigabitEthernet1/2         192.168.1.1     YES manual administratively down down

GigabitEthernet1/3         unassigned      YES unset  administratively down down

GigabitEthernet1/4         unassigned      YES unset  administratively down down

GigabitEthernet1/5         unassigned      YES unset  administratively down down

GigabitEthernet1/6         unassigned      YES unset  administratively down down

GigabitEthernet1/7         unassigned      YES unset  administratively down down

GigabitEthernet1/8         unassigned      YES unset  administratively down down

Internal-Control1/1        127.0.1.1       YES unset  up                    up

Internal-Data1/1           unassigned      YES unset  down                  up

Internal-Data1/2           unassigned      YES unset  up                    up

Internal-Data1/3           unassigned      YES unset  up                    up

Internal-Data1/4           169.254.1.1     YES unset  up                    up

Management1/1              unassigned      YES unset  administratively down up

Enable the Gi1/2 interface using the no shutdown command and verify the interface status. The status and protocol for interface Gi1/2 should be up/up.

ciscoasa(config)# interface Gi1/2

ciscoasa(config-if)# no shut

ciscoasa(config-if)# exit

ciscoasa# show interface ip brief

Interface                  IP-Address      OK? Method Status                Protocol

Virtual0                   127.1.0.1       YES unset  up                    up

GigabitEthernet1/1         unassigned      YES unset  administratively down down

GigabitEthernet1/2         192.168.1.1     YES manual up                    up

GigabitEthernet1/3         unassigned      YES unset  administratively down down

GigabitEthernet1/4         unassigned      YES unset  administratively down down

GigabitEthernet1/5         unassigned      YES unset  administratively down down

GigabitEthernet1/6         unassigned      YES unset  administratively down down

GigabitEthernet1/7         unassigned      YES unset  administratively down down

GigabitEthernet1/8         unassigned      YES unset  administratively down down

Internal-Control1/1        127.0.1.1       YES unset  up                    up

Internal-Data1/1           unassigned      YES unset  down                  up

Internal-Data1/2           unassigned      YES unset  up                    up

Internal-Data1/3           unassigned      YES unset  up                    up

Internal-Data1/4           169.254.1.1     YES unset  up                    up

Management1/1              unassigned      YES unset  administratively down up

Pre-configure outside interface Gi1/1and bring up the interface. You will assign the IP address using ASDM.

ciscoasa(config)# interface Gi1/1

ciscoasa(config-if)# nameif outside

INFO: Security level for "outside" set to 0 by default.

ciscoasa(config-if)# security-level 0

ciscoasa(config-if)# no shut

ciscoasa(config-if)# exit

Test connectivity to the ASA by pinging from PC-B to ASA interface Gi1/2 IP address 192.168.1.1. The pings should be successful.

Configure ASDM and verify access to the ASA.

Configure the ASA to accept HTTPS connections by using the http command to allow access to ASDM from any host on the inside network 192.168.1.0/24.

ciscoasa(config)# http server enable

ciscoasa(config)# http 192.168.1.0 255.255.255.0 inside

Open a browser on PC-B and test the HTTPS access to the ASA by entering https://192.168.1.1.

Note: Be sure to specify the HTTPS protocol in the URL.

Access ASDM and explore the GUI.

After entering the URL above, you should see a security warning about the website security certificate. Click Continue to this website. The ASDM Welcome page will display. From this screen, you can run ASDM as a local application on the PC (installs ASDM on the PC), run ASDM as a browser-based Java applet directly from the ASA, or run the Startup wizard.

Note: You will need to accept all security messages and/or add the ASA IP address to the allowed list of IP addresses in Java.

- If the “Run ASDM” button via Java is not accessible, access your ASA via https://<ip_address>/admin/public/asdm.jnlp to download the JNLP file and then open the file to continue using ASDM.

Click Run ASDM.

Click Yes/Continue/Run in response to any other security warnings.

You should see the Cisco ASDM-IDM Launcher dialog box within which you can enter a username and password. Leave these fields blank and click OK as they have not yet been configured.

      - Click OK to continue. ASDM will load the current configuration into the GUI.

Click Cancel on the “Cannot Connect” FirePOWER module window to continue.

The initial GUI screen is displayed with various areas and options. The menu at the top left of the screen contains three main sections: Home, Configuration, and Monitoring. The Home section is the default and has three dashboards: Device,Firewall, and ASA FirewPOWER status. The Device dashboard is the default screen and shows device information, such as Type (ASA 5506), ASA and ASDM version, the amount of memory, and firewall mode (routed). There are five areas on the Device dashboard:

Device Information

Interface Status

VPN Summary

System Resources Status

Traffic Status

Click the Configuration and Monitoring buttons to become familiar with their layout and to see what options are available.

Configure Basic ASA Settings and Firewall Using the ASDM Startup Wizard

Access the Configuration menu and launch the Startup wizard.

On the menu bar, click Configuration. There are five main configuration areas:

Device Setup

Firewall

Remote Access VPN

Site-to-Site VPN

Device Management

The Device Setup Startup wizard is the first option available and displays by default. Read through the on-screen text describing the Startup wizard, and then click Launch Startup Wizard.

Configure hostname, domain name, and the enable password.

On the first Startup Wizard screen, modify the existing configuration or reset the ASA to the factory defaults. Ensure that the Modify Existing Configuration option is selected, and click Next to continue.

On the Startup Wizard Step 2 screen, configure the ASA hostname CCNAS-ASA and domain name ccnasecurity.com.

Click the check box for changing the enable mode password, change it from blank (no password) to cisco12345, and enter it again to confirm.

When the entries are completed, click Next to continue.

Configure the inside and outside interfaces.

On the Startup Wizard Step 3 screen for the Outside and Inside Interfaces, do not change the current settings because these were previously defined using the CLI. 
The inside Gi1/2 interface is named inside, and the security level is set to 100 (highest).
The Outside interface Gi1/1 is named outside, and the security level is set to 0 (lowest).

Enter an Outside IP Address of 209.165.200.226 and a Mask of 255.255.255.248. You can use the pull-down menu to select the mask. 
Leave the inside interface IP address as 192.168.1.1 with a mask of 255.255.255.0.

Click Next to continue.

Configure Other Interface Configurations.

On the Startup Wizard Step 4 screen – view the current ASA 5506 interface configurations. Click Next to continue.

View Static Routes

On the Startup Wizard Step 5 screen – view the configuration screen for IPv4 and IPv6 Static Routes. Click Next to continue.

Configure DHCP, address translation, and administrative access.

On the Startup Wizard Step 6 screen – DHCP Server, click the Enable DHCP server on the inside interface check box.

Enter a Starting IP Address of 192.168.1.31 and an Ending IP Address of 192.168.1.39.

Enter the DNS Server 1 address of 10.20.30.40 and the Domain Name ccnasecurity.com. 
Do NOT check the box to Enable auto-configuration from interface.

Click Next to continue.

On the Startup Wizard Step 7 of 12– Address Translation (NAT/PAT)

- Click Use Port Address Translation (PAT). 
The default is to use the IP address of the outside interface.

Note: You can also specify a particular IP address for PAT or a range of addresses with NAT. 
Click Next to continue.

On the Startup Wizard Step 8 of 12– Administrative Access, HTTPS/ASDM access is currently configured for hosts on the inside network 192.168.1.0/24.

- -Add SSH access to the ASA for the inside network 192.168.1.0 with a subnet mask of 255.255.255.0. 

-Add SSH access to the ASA from host 172.16.3.3 on the outside network. 

-Ensure that the Enable HTTP server for HTTPS/ASDM access check box is selected.

- -Click Next to continue.

Bypass the ASA FirePOWER Configuration

On the Startup Wizard Step 9 of 12 – ASA FirePower Basic Configuration, click the checkbox “Select to Bypass ASA FirePOWER Configuration and click Next.

View the Auto Update Server settings.

On the Startup Wizard Step 10– Auto Update Server, review the options and click Next. 
Do not modify any settings here.

Cisco Smart Call Home Enrollment

On the Cisco Smart Call Home Enrollment Step 11, review the options and click Next. 
Do not modify any settings here.

Review the summary and deliver the commands to the ASA.

On the Startup Wizard Step 12 screen – Startup Wizard Summary, review the Configuration Summary and click Finish. 
ASDM will deliver the commands to the ASA device and then reload the modified configuration.

Note: If a GUI dialogue box appears requesting a username and password, go to the command line on the ASA and create a username of admin01 with password admin01pass. Use this to authenticate in the GUI dialogue box.

Close the ASDM and if asked, save the configuration.

Run ASDM again from the web browser on PC-B. 
The new enable password is cisco12345 with no username to access ASDM.

Return to the Device dashboard and check the Interface Status window. 
You should see the inside and outside interfaces with IP address and status. 
The inside interface should show a number of Kb/s.

Test access to an external website from PC-B.

Open a browser on PC-B and enter the IP address of the R1 G0/0 interface (209.165.200.225) to simulate access to an external website.

The R1 HTTP server was enabled in Part 1. You should be prompted with a user authentication login dialog box from the R1 GUI device manger.

Enter the username admin01 and the password admin01pass.

Exit the browser. 
You should see TCP activity in the ASDM Device Dashboard Traffic Status window on the Home page.

Test access to an external website using the ASDM Packet Tracer utility.

Within the ASDM Device, click Tools > Packet Tracer.

Select the inside interface from the Interface drop-down list and click TCP from the Packet Type radio buttons.

From the Source drop-down list, select IP Address and enter the address 192.168.1.3 (PC-B) with a Source Port of 1500. From the Destination drop-down list, select IP Address, and enter 209.165.200.225 (R1 Gi0/0) with a Destination Port of 80.

Click Start to begin the trace of the packet. The packet should be permitted.

Click Clear to reset the entries.

Try another trace and select outside from the Interface drop-down list and leave TCP as the packet type.

From the Sources drop-down list, select IP Address, and enter 209.165.200.225 (R1 G0/0) and a Source Port of 1500. From the Destination drop-down list, select IP Address and enter the address 209.165.200.226 (ASA outside interface) with a Destination Port of telnet.

Click Start to begin the trace of the packet. The packet should be dropped. Click Close to continue.

Configure ASA Settings from the ASDM Configuration Menu

In Part 4, you will set the ASA clock, configure a default route, test connectivity using the ASDM tools ping and traceroute, configure local AAA user authentication, test SSH access, and modify the MPF application inspection policy.

Set the ASA date and time.

On the Configuration screen > Device Setup menu, click System Time > Clock.

Select your Time Zone from the drop-down list and enter the current date and time in the fields provided. (The clock is a 24-hour clock.)

Click Apply to send the commands to the ASA.

Configure a static default route for the ASA.

On the ASDM Tools menu, select Ping and enter the IP address of router R1 S0/0/0 (10.1.1.1).

Click Ping

The ASA does not have a default route to unknown external networks. The ping should fail because the ASA does not have a route to 10.1.1.1.

Click Close to continue.

From the Configuration screen > Device Setup menu, click Routing > Static Routes.

Click IPv4 Only and click Add to add a new static route.

On the Add Static Route dialog box, select the outside interface from the drop-down list.

Click the ellipsis button to the right of Network and select any4 from the list of network objects andd click OK. The selection of any4 translates to a “quad zero” route.

For the Gateway IP, enter 209.165.200.225 (R1 G0/0).

Click OK and then Apply to send the commands to the ASA.

z

On the ASDM Tools menu, select Ping and enter the IP address of router R1 S0/0/0 (10.1.1.1). The ping should succeed this time. Click Close to continue.

On the ASDM Tools menu, select Traceroute and enter the IP address of external host PC-C (172.16.3.3).

Click Trace Route. The traceroute should succeed and show the hops from the ASA through R1, R2, and R3 to host PC-C.

Click Close to continue.

Configure AAA user authentication using the ASA local database.

Enable AAA user authentication to access the ASA using SSH. You allowed SSH access to the ASA from the inside network and the outside host PC-C when the Startup wizard was run. To allow the administrator to have SSH access to the ASA, you will create a user in the local database.

On the Configuration screen > Device Management area, click Users/AAA.

Click User Accounts

Click Add.

Create a new user named admin02 with a password of admin02pass and enter the password again to confirm it. Allow this user Full access (ASDM, SSH, Telnet, and console) and set the privilege level to 15.

Click OK to add the user and click Apply to send the command to the ASA.

On the Configuration screen > Device Management area, click Users/AAA. Click AAA Access.

On the Authentication tab, click the check box to require authentication for HTTP/ASDM and SSH connections and specify the LOCAL server group for each connection type.

Click Apply to send the commands to the ASA.

Note: The next action you attempt within ASDM will require that you log in as admin02 with the password admin02pass.

Test SSH access to the ASA.

Open a SSH client on PC-B, such as PuTTY and connect to the ASA inside interface at IP address 192.168.1.1. When prompted to log in, enter the user name admin02 and the password admin02pass.

From PC-C, open an SSH client, such as PuTTY, and attempt to access the ASA outside interface at 209.165.200.226. When prompted to log in, enter the user name admin02 and the password admin02pass.

After logging in to the ASA using SSH, enter the enable command and provide the password cisco12345. Issue the show run command to display the current configuration that you have created using ASDM.

Modify the MPF application inspection policy.

For application layer inspection, and other advanced options, the Cisco Modular Policy Framework (MPF) is available on ASAs.

The default global inspection policy does not inspect ICMP. To enable hosts on the internal network to ping external hosts and receive replies, ICMP traffic must be inspected.

Within the ASDM on PC-B, access the Configuration tab > Firewall area menu, click Service Policy Rules.

Select the inspection_default policy and click Edit to modify the default inspection rules.

On the Edit Service Policy Rule window, click the Rule Actions tab and select the ICMP check box. Do not change the other default protocols that are checked.

Click OK > Apply to send the commands to the ASA. 
If prompted, log in as admin02 with the password admin02pass.

From PC-B, ping the external interface of R1 S0/0/0 (10.1.1.1). The pings should be successful.

Configure DMZ, Static NAT, and ACLs

In this part, you will create a DMZ on the ASA, configure static NAT to a DMZ server, and apply an ACL to control access to the server.

Configure the ASA DMZ VLAN 3 interface.

In this step, you will configure interface Gi1/3 named dmz, set the security level to 70, and limit communication from this interface to the inside (Gi1/2) interface.

On the Configuration screen > Device Setup menu, click Interface Settings > Interfaces. The currently defined inside and outside interfaces are listed.

Double-click GigabitEthernet1/3 to configure the dmz interface.

The Edit Interface dialog box will open. In the Interface Name box, name the interface dmz, assign it a security level of 70, and make sure the Enable Interface checkbox is checked.

Ensure that the Use Static IP option is selected and enter an IP address of 192.168.2.1 with a subnet mask of 255.255.255.0.

Click OK to close the window.
When prompted on the Security Level Change, click OK.

You should see the new interface named dmz, in addition to the inside and outside interfaces.

Check the box Enable traffic between two or more interfaces which are configured with the same security levels.

Click Apply to send the commands to the ASA.

Configure the DMZ server and static NAT.

To accommodate the addition of a DMZ and a web server, you will use another address from the ISP range assigned, 209.165.200.224/29 (.224-.231). R1 G0/0 and the ASA outside interface are already using 209.165.200.225 and .226. You will use public address 209.165.200.227 and static NAT to provide address translation access to the server.

On the Firewall menu, click the Public Servers option and click Add to define the DMZ server and services offered.

In the Add Public Server dialog box, specify the Private Interface as dmz, the Public Interface as outside, and the Public IP address as 209.165.200.227.

Click the ellipsis button to the right of Private IP Address. In the Browse Private IP Address window, click Add to define the server as a Network Object. Enter the name DMZ-Server, select Host from the Type pull-down menu, enter the IP Address 192.168.2.3, and a Description of PC-A.

Click OK

From the Browse Private IP Address window, verify that the DMZ-Server appears in the Selected Private IP Address field and click OK. You will return to the Add Public Server dialog box.

Click the ellipsis button to the right of Private Service.

Double-click to select the following services: tcp/ftp, tcp/http, icmp/echo, and icmp/echo-reply

Click OK to continue and return to the Add Public Server dialog.

Note: You can specify Public services if they are different from the Private services, using the option on this screen.

When you have completed all the information in the Add Public Server dialog box, it should look like the one shown below. Click OK to add the server.

Click Apply at the Public Servers screen to send the commands to the ASA.

View the DMZ Access Rule generated by ASDM.

After the creation of the DMZ server object and selection of services, ASDM automatically generates an Access Rule (ACL) to permit the appropriate access to the server and applies it to the outside interface in the incoming direction.

View this ACL in ASDM by clicking Configuration > Firewall > Access Rules. It appears as an outside incoming rule. You can select the rule and use the horizontal scroll bar to see all of the components.

Note: You can also see the commands generated by using the Tools > Command Line Interface and entering the show run command.

Test access to the DMZ server from the outside network.

From PC-C, ping the IP address of the static NAT public server address (209.165.200.227). The pings should be successful.

Because the ASA inside interface Gi1/2 is set to security level 100 (the highest) and the DMZ interface Gi1/3 is set to 70, you can also access the DMZ server from a host on the inside network. The ASA acts like a router between the two networks. Ping the DMZ server (PC-A) internal address (192.168.2.3) from inside network host PC-B (192.168.1.X). The pings should be successful due to the interface security level and the fact that ICMP is being inspected on the inside interface by the global inspection policy.

The DMZ server cannot ping PC-B on the inside network. This is because the DMZ interface Gi1/3 has a lower security level. Try to ping from the DMZ server PC-A to PC-B at the IP address 192.168.1.3. The pings should Not be successful.

Use ASDM Monitoring to graph packet activity.

There are a number of aspects of the ASA that can be monitored using the Monitoring screen. The main categories on this screen are Interfaces, VPN, Routing, Properties, and Logging. In this step, you will create a graph to monitor packet activity for the outside interface.

On the Monitoring screen > Interfaces menu, click Interface Graphs > outside. Select Packet Counts and click Add to add the graph. The exhibit below shows Packet Counts added.

Click Show Graphs to display the graph. Initially, there is no traffic displayed.

From a privileged mode command prompt on R2, simulate Internet traffic to the ASA by pinging the DMZ server’s public address with a repeat count of 1000. You can increase the number of pings if desired.

R2# ping 209.165.200.227 repeat 1000

Type escape sequence to abort.

Sending 1000, 100-byte ICMP Echos to 209.165.200.227, timeout is 2 seconds:

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

<output omitted>

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!

Success rate is 100 percent (1000/1000), round-trip min/avg/max = 1/2/12 ms

You should see the results of the pings from R2 on the graph. The scale of the graph is automatically adjusted depending on the volume of traffic. You can also view the data in tabular form by clicking the Table tab. Notice that the View selected at the bottom left of the Graph screen is Real-time, data every 10 seconds. Click the pull-down list to see the other available options.

Ping from PC-B to R1 S0/0/0 at 10.1.1.1 using the –n option (number of packets) to specify 100 packets.

C:>\ ping 10.1.1.1 –n 100

Note: The response from the PC is relatively slow, and it may take a while to show up on the graph as Output Packet Count. The graph below shows an additional 4000 input packets and both input and output packet counts.

Reflection

What are some of the benefits of using ASDM over the CLI?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

The ASDM GUI is easier to use, especially for less technical staff, and can generate very complex configurations through the use of mouse selections, fill-in fields, and wizards.

What are some of the benefits of using the CLI over ASDM?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

In some cases, the CLI can provide more precise control over the desired configuration. Also, some CLI commands are necessary to prepare the ASA for GUI access. CLI requires only a serial console connection, whereas ASDM requires Layer 3 (IP) connectivity to an ASA interface.

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |

Device Configs

ASA 5506

CCNAS-ASA# show run

: Saved

:

: Serial Number: JAD2002064E

: Hardware:   ASA5506W, 4096 MB RAM, CPU Atom C2000 series 1250 MHz, 1 CPU (4 cores)

:

ASA Version 9.8(2)

!

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password $sha512$5000$Rx0o0uDtDHW3HJg5GqrtLA==$H0B/0cTgFkmdPrSiKD9xCg== pbkdf2

xlate per-session deny tcp any4 any4

xlate per-session deny tcp any4 any6

xlate per-session deny tcp any6 any4

xlate per-session deny tcp any6 any6

xlate per-session deny udp any4 any4 eq domain

xlate per-session deny udp any4 any6 eq domain

xlate per-session deny udp any6 any4 eq domain

xlate per-session deny udp any6 any6 eq domain

names

!

interface GigabitEthernet1/1

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

!

interface GigabitEthernet1/2

nameif inside

security-level 100

ip address 192.168.1.1 255.255.255.0

!

interface GigabitEthernet1/3

nameif dmz

security-level 70

ip address 192.168.2.1 255.255.255.0

!

interface GigabitEthernet1/4

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/5

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/6

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/7

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/8

shutdown

no nameif

no security-level

no ip address

!

interface Management1/1

management-only

shutdown

no nameif

no security-level

no ip address

!

ftp mode passive

clock timezone CST -6

clock summer-time CDT recurring

dns server-group DefaultDNS

domain-name ccnasecurity.com

same-security-traffic permit inter-interface

object network DMZ-Server

host 192.168.2.3

description PC-A

object-group service DM_INLINE_SERVICE_0

service-object icmp echo

service-object icmp echo-reply

service-object tcp destination eq ftp

service-object tcp destination eq www

access-list outside_access extended permit object-group DM_INLINE_SERVICE_0 any4 object DMZ-Server

pager lines 24

mtu inside 1500

mtu outside 1500

mtu dmz 1500

icmp unreachable rate-limit 1 burst-size 1

no asdm history enable

arp timeout 14400

no arp permit-nonconnected

arp rate-limit 16384

!

object network DMZ-Server

nat (dmz,outside) static 209.165.200.227

!

nat (inside,outside) after-auto source dynamic any interface

access-group outside_access in interface outside

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

timeout xlate 3:00:00

timeout pat-xlate 0:00:30

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 sctp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout sip-provisional-media 0:02:00 uauth 0:05:00 absolute

timeout tcp-proxy-reassembly 0:01:00

timeout floating-conn 0:00:00

timeout conn-holddown 0:00:15

timeout igp stale-route 0:01:10

user-identity default-domain LOCAL

aaa authentication http console LOCAL

aaa authentication ssh console LOCAL

aaa authentication login-history

http server enable

http 192.168.1.0 255.255.255.0 inside

no snmp-server location

no snmp-server contact

service sw-reset-button

crypto ipsec security-association pmtu-aging infinite

crypto ca trustpool policy

telnet timeout 5

ssh stricthostkeycheck

ssh 192.168.1.0 255.255.255.0 inside

ssh 172.16.3.3 255.255.255.255 outside

ssh timeout 5

ssh key-exchange group dh-group1-sha1

console timeout 0

dhcpd address 192.168.1.31-192.168.1.39 inside

dhcpd dns 10.20.30.40 interface inside

dhcpd domain ccnasecurity.com interface inside

dhcpd enable inside

!

threat-detection basic-threat

threat-detection statistics access-list

no threat-detection statistics tcp-intercept

dynamic-access-policy-record DfltAccessPolicy

username admin01 password $sha512$5000$Gg8bc2p76xot6DChUgS1Qg==$sXPawOa7sXjlHJYhPjprEQ== pbkdf2

username admin02 password $sha512$5000$6tGjwQsB/ok8dtSiC8wRig==$qX0TNOQVG0vTQ4ljNl3OVQ== pbkdf2 privilege 15

!

class-map inspection_default

match default-inspection-traffic

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map global_policy

class inspection_default

inspect ftp

inspect h323 h225

inspect h323 ras

inspect ip-options

inspect netbios

inspect rsh

inspect rtsp

inspect skinny

inspect esmtp

inspect sqlnet

inspect sunrpc

inspect tftp

inspect sip

inspect xdmcp

inspect dns preset_dns_map

inspect icmp

policy-map type inspect dns migrated_dns_map_2

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map type inspect dns migrated_dns_map_1

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

!

service-policy global_policy global

prompt hostname context

no call-home reporting anonymous

call-home

profile CiscoTAC-1

no active

destination address http https://tools.cisco.com/its/service/oddce/services/DDCEService

destination address email callhome@cisco.com

destination transport-method http

subscribe-to-alert-group diagnostic

subscribe-to-alert-group environment

subscribe-to-alert-group inventory periodic monthly

subscribe-to-alert-group configuration periodic monthly

subscribe-to-alert-group telemetry periodic daily

Cryptochecksum:53a2d4eef0d9e361d2a39804fea84151

: end

Router R1

R1#show run

Building configuration...

Current configuration : 2120 bytes

!

! Last configuration change at 13:01:33 UTC Tue Jan 6 2015

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$iMudc40vBlrXcE$yHS3ffCML2lsFdpjPHMLQW4FUDvFWjh/wDmVdCnlWbY

!

no aaa new-model

memory-size iomem 15

!

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

voice-card 0

!

license udi pid CISCO2911/K9 sn FTX1713ALKC

license accept end user agreement

license boot module c2900 technology-package securityk9

license boot module c2900 technology-package uck9

license boot module c2900 technology-package datak9

!

username admin01 privilege 15 secret 9 $9$A.ebsSc7e2WHlE$VttN77bA8jO4A7xkL4AqXqekmdyXUJV/WU1fuYrIMfs

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 209.165.200.225 255.255.255.248

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 10.1.1.1 255.255.255.252

clock rate 128000

!

interface Serial0/0/1

no ip address

shutdown

!

ip forward-protocol nd

!

ip http server

ip http authentication local

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 10.1.1.2

!

control-plane

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

gatekeeper

shutdown

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 5 0

logging synchronous

login local

transport input ssh

!

scheduler allocate 20000 1000

!

end

Router R2

R2#show run

Building configuration...

Current configuration : 1805 bytes

!

! Last configuration change at 08:38:04 UTC Thu Nov 30 2017

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R2

!

boot-start-marker

boot-end-marker

!

no aaa new-model

memory-size iomem 15

!

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

voice-card 0

!

license udi pid CISCO2911/K9 sn FTX1713ALJP

license boot module c2900 technology-package securityk9

license boot module c2900 technology-package uck9

license boot module c2900 technology-package datak9

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 10.1.1.2 255.255.255.252

!

interface Serial0/0/1

ip address 10.2.2.2 255.255.255.252

clock rate 128000

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 172.16.3.0 255.255.255.0 10.2.2.1

ip route 209.165.200.224 255.255.255.248 10.1.1.1

!

control-plane

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

gatekeeper

shutdown

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end

Router R3

R3#show run

Building configuration...

Current configuration : 1818 bytes

!

! Last configuration change at 12:35:22 UTC Tue Jan 6 2015

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R3

!

boot-start-marker

boot-end-marker

!

no aaa new-model

memory-size iomem 15

!

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

voice-card 0

!

license udi pid CISCO2911/K9 sn FTX1713ALJV

license accept end user agreement

license boot module c2900 technology-package securityk9

license boot module c2900 technology-package uck9

license boot module c2900 technology-package datak9

!

vtp domain TSHOOT

vtp mode transparent

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 172.16.3.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

clock rate 125000

!

interface Serial0/0/1

ip address 10.2.2.1 255.255.255.252

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 10.2.2.2

!

control-plane

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

gatekeeper

shutdown

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end

Switches S1, S2, and S3 – Use default configs, except for host name