CCNA Security

Chapter 10 – Configure Clientless Remote Access SSL VPNs Using ASDM (Instructor Version)

(ASA-5506 / Equiv)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

|  | This lab has been updated for use on NETLAB+. www.netdevgroup.com |
| --- | --- |

Topology

Note: ISR G1 devices use FastEthernet interfaces instead of GigabitEthernet Interfaces.

IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 209.165.200.225 | 255.255.255.248 | N/A | ASA Gi1/1 |
|  | S0/0/0 (DCE) | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
|  | S0/0/1 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 172.16.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| ASA | Gi1/2 | 192.168.1.1 | 255.255.255.0 | NA | S2 F0/24 |
|  | Gi1/1 | 209.165.200.226 | 255.255.255.248 | NA | R1 G0/0 |
|  | Gi1/3 | 192.168.2.1 | 255.255.255.0 | NA | S1 F0/24 |
| PC-A | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S1 F0/6 |
| PC-B | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S2 F0/18 |
| PC-C | NIC | 172.16.3.3 | 255.255.255.0 | 172.16.3.1 | S3 F0/18 |

Objectives

Part 1: Basic Router/Switch/PC Configuration

Configure basic settings for routers.

Configure PC host IP settings.

Verify connectivity.

Save the basic running configuration for each router and switch.

Part 2: Access the ASA Console and ASDM

Access the ASA console.

Clear the previous ASA configuration settings.

Bypass Setup mode.

Configure the ASA by using the CLI script.

Access ASDM.

Part 3: Configuring Clientless SSL VPN Remote Access Using ASDM

Start the VPN wizard.

Configure the SSL VPN user interface.

Configure AAA user authentication.

Configure the VPN group policy.

Configure a bookmark list (clientless connections only).

Review the configuration summary and deliver the commands to the ASA.

Verify the ASDM SSL VPN connection profile.

Verify VPN access from the remote host.

Access the web portal page.

View the clientless remote user session using the ASDM Monitor.

Background / Scenario

In addition to stateful firewall and other security features, the ASA can provide both site-to-site and remote access VPN functionality. The ASA provides two main deployment modes that are found in Cisco SSL remote access VPN solutions:

Clientless SSL VPN—Clientless, browser-based VPN that lets users establish a secure, remote-access VPN tunnel to the ASA using a web browser and built-in SSL to protect VPN traffic. After authentication, users are presented with a portal page and can access specific, predefined internal resources from the portal.

Client-Based SSL VPN—Provides full-tunnel SSL VPN connection, but requires a VPN client application to be installed on the remote host. After authentication, users can access any internal resource as if they were physically on the local network. The ASA supports both SSL and IPsec client-based VPNs.

In Part 1 of this lab, you will configure the topology and non-ASA devices. In Part 2, you will prepare the ASA for ASDM access. In Part 3, you will use the ASDM VPN wizard to configure a clientless SSL remote access VPN and verify access using a remote PC with a browser.

Your company has two locations connected to an ISP. Router R1 represents a CPE device managed by the ISP. Router R2 represents an intermediate Internet router. Router R3 connects users at the remote branch office to the ISP. The ASA is an edge security device that connects the internal corporate network and DMZ to the ISP while providing NAT services to inside hosts.

Management has asked you to provide VPN access, using the ASA as a VPN concentrator, to teleworkers. They want you to test the clientless access model, using SSL and a browser for client access.

Note: The router commands and output in this lab are from a Cisco 1941 router with Cisco IOS Release 15.4(3)M2 (with a Security Technology Package license). Other routers and Cisco IOS versions can be used. See the Router Interface Summary Table at the end of the lab to determine which interface identifiers to use based on the equipment in the lab. Depending on the router model and Cisco IOS version, the commands available and the output produced might vary from what is shown in this lab.

The ASA used with this lab is a Cisco model 5506 with an 8-port integrated router, running OS version 9.8(1), Adaptive Security Device Manager (ASDM) version 7.8(1), and comes with a Base license.

Instructor Note: Instructions for erasing switches and routers are provided in Chapter 0.0.0.0. Instructions for erasing the ASA, accessing the console, and accessing ASDM are provided in this lab.

Basic Router/Switch/PC Configuration

In Part 1, you will configure basic settings on the routers such as interface IP addresses and static routing.

Note: Do not configure any ASA settings at this time.

Configure R1 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R1. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

Note: Depending on the router model, interfaces might be numbered differently than those listed. You might need to alter the designations accordingly.

Note: Passwords in this task are set to a minimum of 10 characters but are relatively simple for the benefit of performing the lab. More complex passwords are recommended in a production network.

enable

config t

hostname R1

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

logging synchronous

exit

interface gigabitethernet 0/0

ip address 209.165.200.225 255.255.255.248

no shut

exit

int serial 0/0/0

ip address 10.1.1.1 255.255.255.252

clock rate 2000000

no shut

exit

ip route 0.0.0.0 0.0.0.0 Serial0/0/0

crypto key generate rsa general-keys modulus 1024

Configure R2 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R2. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

enable

config t

hostname R2

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

logging synchronous

exit

interface serial 0/0/0

ip address 10.1.1.2 255.255.255.252

no shut

exit

interface serial 0/0/1

ip address 10.2.2.2 255.255.255.252

clock rate 2000000

no shut

exit

ip route 209.165.200.224 255.255.255.248 Serial0/0/0

ip route 172.16.3.0 255.255.255.0 Serial0/0/1

crypto key generate rsa general-keys modulus 1024

Configure R3 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R3. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

enable

config t

hostname R3

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input

exec-timeout 5 0

logging synchronous

exit

interface gigabitethernet 0/1

ip address 172.16.3.1 255.255.255.0

no shut

exit

int serial 0/0/1

ip address 10.2.2.1 255.255.255.252

no shut

exit

ip route 0.0.0.0 0.0.0.0 Serial0/0/1

crypto key generate rsa general-keys modulus 1024

    - Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A, PC-B, and PC-C as shown in the IP Addressing table.

    - Verify connectivity.

Because the ASA is the focal point for the network zones and it has not yet been configured, there will be no connectivity between devices that are connected to it. However, PC-C should be able to ping the R1 interface G0/0. From PC-C, ping the R1 G0/0 IP address (209.165.200.225). If these pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Note: If you can ping from PC-C to R1 G0/0 and S0/0/0, you have demonstrated that static routing is configured and functioning correctly.

    - Save the basic running configuration for each router and switch.

Accessing the ASA Console and ASDM

Clear the previous ASA configuration settings.

Use the write erase command to remove the startup-config file from flash memory.

Note: The erase startup-config IOS command is not supported on the ASA.

Use the reload command to restart the ASA. This causes the ASA to display in CLI Setup mode. If you see the System config has been modified. Save? [Y]es/[N]o: message, type n, and press Enter.

Bypass Setup mode.

When the ASA completes the reload process, it should detect that the startup configuration file is missing and go into Setup mode. If it does not come up in this mode, repeat Step 1.

When prompted to preconfigure the firewall through interactive prompts (Setup mode), respond with no.

Enter privileged EXEC mode with the enable command. The password should be blank (no password).

Configure the ASA by using the CLI script.

In this step, you will use a CLI script to configure basic settings, the firewall and DMZ.

Other than the defaults that the ASA automatically inserts, use the show run command to confirm that there is no previous configuration in the ASA.

Enter global configuration mode. When prompted to enable anonymous call-home reporting, respond no.

Copy and paste the Pre-VPN Configuration Script commands listed below at the ASA global configuration mode prompt to start configuring the SSL VPNs.

- Observe the messages as the commands are applied to ensure that there are no warnings or errors. If prompted to replace the RSA key pair, respond yes.

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password cisco12345

!

interface Gi1/1

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

no shut

!

interface Gi1/2

nameif inside

security-level 100

ip address 192.168.1.1 255.255.255.0

no shut

!

interface Gi1/3

nameif dmz

security-level 70

ip address 192.168.2.1 255.255.255.0

no shut

!

object network inside-net

subnet 192.168.1.0 255.255.255.0

!

object network dmz-server

host 192.168.2.3

!

access-list OUTSIDE-DMZ extended permit ip any host 192.168.2.3

!

object network inside-net

nat (inside,outside) dynamic interface

!

object network dmz-server

nat (dmz,outside) static 209.165.200.227

!

access-group OUTSIDE-DMZ in interface outside

!

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

!

username admin01 password admin01pass

!

aaa authentication telnet console LOCAL

aaa authentication ssh console LOCAL

aaa authentication http console LOCAL

!

http server enable

http 192.168.1.0 255.255.255.0 inside

ssh 192.168.1.0 255.255.255.0 inside

telnet 192.168.1.0 255.255.255.0 inside

telnet timeout 10

ssh timeout 10

!

class-map inspection_default

match default-inspection-traffic

policy-map global_policy

class inspection_default

inspect icmp

!

crypto key generate rsa modulus 1024

At the privileged EXEC mode prompt, issue the write mem (or copy run start) command to save the running configuration to the startup configuration and the RSA keys to non-volatile memory.

Access ASDM.

Open a browser on PC-B and test the HTTPS access to the ASA by entering https://192.168.1.1. After entering the https://192.168.1.1 URL, you should see a security warning about the website security certificate. Click Continue to this website. Click Yes for any other security warnings.

Note: Specify the HTTPS protocol in the URL.

      - At the ASDM welcome page, click Run ASDM. The ASDM-IDM Launcher will display. Log in as user admin01 with password admin01pass.

Note: You will need to accept all security messages and/or add the ASA IP address to the allowed list of IP addresses in Java.

- If the “Run ASDM” button via Java is not accessible, access your ASA via https://<ip_address>/admin/public/asdm.jnlp to download the JNLP file and then open the file to continue using ASDM.

Configuring Clientless SSL VPN Remote Access Using ASDM

Start the VPN wizard.

On the ASDM main menu, click Wizards > VPN Wizards > Clientless SSL VPN Wizard. The Clientless SSL VPN Connection screen displays.

Review the on-screen text and topology diagram, and then click Next to continue.

Configure the SSL VPN user interface.

On the SSL VPN Interface screen, configure SSL-VPN as the Connection Profile Name, and specify outside as the interface to which outside users will connect.

Note: By default, the ASA uses a self-signed certificate to send to the client for authentication. Optionally, the ASA may be configured to use a third-party certificate that is purchased from a well-known certificate authority, such as VeriSign, to connect clients. In the event that a certificate is purchased, it may be selected in the Digital Certificate drop-down menu.

The SSL VPN Interface screen provides links in the Information section. These links identify the URLs that need to be used for the SSL VPN service access (log in) and for Cisco ASDM access (to access the Cisco ASDM software).

Click Next to continue.

Configure AAA user authentication.

On the User Authentication screen, click Authenticate using the local user database.

Enter the user name SSL-VPN-USER with password cisco12345.

Click Add to create the new user and click Next to continue.

Configure the VPN group policy.

On the Group Policy screen, create a new group policy named SSL-VPN-POLICY. (When configuring a new policy, the policy name cannot contain any spaces.)

Click Next to continue.

Configure the bookmark list (clientless connections only).

A bookmark list is a set of URLs configured to be used in the clientless SSL VPN web portal.
If there were bookmarks already listed, you would use the Bookmark List drop-down list, select the bookmark of choice, and click Next to continue with the SSL VPN wizard.

Note: There are no configured bookmark lists by default and, therefore, they must be configured by the network administrator.

On the Clientless Connections Only – Bookmark List screen, click Manage to create an HTTP server bookmark in the bookmark list.

In the Configure GUI Customization Objects window, click Add to open the Add Bookmark List window. Name the list Web-Server.

- Note: If the Web-Server bookmark list is shown as available from a previous configuration, you can delete it in ASDM and re-create it.

In the Add Bookmark List window, click Add to open the Select Bookmark Type window.

As shown in the figure, the ASDM can create three types of bookmarks. Select the URL with GET or POST method, click OK.

Enter the bookmark title and enter the server destination IP address or hostname as the URL to be used with the bookmark entry. In this example, the Bookmark Title of Web-Mail is entered and an internal IP address of 192.168.2.3 (the DMZ server) is specified. If this server has HTTP web services with web mail installed and functional, the outside users are able to access the server from the ASA portal when they connect.

Click OK to continue and return to the Add Bookmark List window which now displays the Web-Server bookmark title and URL.

Click OK to continue and return to the Configure GUI Customization Objects window which now displays the Web-Server bookmark.

Click OK to continue and return to the Bookmark List window and click Next to continue.

Review the configuration summary and deliver the commands to the ASA.

The Summary page is displayed next. Verify that the information configured in the SSL VPN wizard is correct. Click Back to make changes, or click Cancel and restart the VPN wizard. Click Finish to complete the process and deliver the commands to the ASA

Verify the ASDM SSL VPN connection profile.

In ASDM, click Configuration > Remote Access VPN > Clientless SSL VPN Access > Connection Profiles. In this window, the VPN configuration can be verified and edited.

Verify VPN access from the remote host.

Open the browser on PC-C and enter the login URL for the SSL VPN into the address field (https://209.165.200.226). Use secure HTTP (HTTPS) because SSL is required to connect to the ASA.

- Note: If you encounter a prompt stating that the connection is not trusted or secure, accept the self-signed certificate to continue.

The Logon window should display. Enter the previously configured username SSL-VPN-USER and password cisco12345, and click Logon to continue.

Note: The ASA may request confirmation that this is a trusted site. If requested, click Yes to proceed.

Note: If the AnyConnect client fails authentication, reset the password.

Configuration > Remote Access VPN > Local Users > SSL-VPN-USER > Edit > Identity > Change user password (retype in the correct username and password combo). Click OK followed by clicking Apply.

Access the web portal window.

After the user authenticates, the ASA SSL web portal page lists the various bookmarks previously assigned to the profile. If the Bookmark points to a valid server IP address or hostname that has HTTP web services installed and functional, the outside user will be able to access the server from the ASA portal.

Note: In this lab, the web mail server is not installed.

View the clientless remote user session using the ASDM Monitor.

While the remote user at PC-C is still logged in and on the ASA portal page, you can view the session statistics using ASDM monitor.

From the ASDM menu bar on PC-B, click Monitoring and then select VPN > VPN Statistics > Sessions. Click the Filter By pull-down list and select Clientless SSL VPN. You should see the SSL-VPN-USER session logged in from PC-C (172.16.3.3).

Note: You may need to click Refresh to display the remote user session.

Log out of the web portal page.

The user should log out of the web portal window on PC-C using the Logout button when done. However, the web portal will also time out if there is no activity. In either case a logout window will be displayed informing users that for additional security, they should clear the browser cache, delete the downloaded files, and close the browser window.

Reflection

What are some benefits of clientless vs. client-based VPNs?

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

They are easier to setup because only a browser is required and no client software needs to be installed. They can be used to limit access to very specific resources based on URLs that are defined by network administration.

What are some differences when using SSL as compared to IPsec for remote access tunnel encryption?

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

Client-based VPNs can offer a more secure tunnel, if using IPsec, but are somewhat more complex to configure.

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |

Device Configs

ASA 5506 Final Config

CCNAS-ASA# show run

: Saved

:

: Serial Number: JAD2002064E

: Hardware:   ASA5506W, 4096 MB RAM, CPU Atom C2000 series 1250 MHz, 1 CPU (4 cores)

:

ASA Version 9.8(2)

!

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password $sha512$5000$CQjFkol8u9efEcwrY9udDQ==$HeOdwc6BbBO/yKUnl4mUfw== pbkdf2

xlate per-session deny tcp any4 any4

xlate per-session deny tcp any4 any6

xlate per-session deny tcp any6 any4

xlate per-session deny tcp any6 any6

xlate per-session deny udp any4 any4 eq domain

xlate per-session deny udp any4 any6 eq domain

xlate per-session deny udp any6 any4 eq domain

xlate per-session deny udp any6 any6 eq domain

names

!

interface GigabitEthernet1/1

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

!

interface GigabitEthernet1/2

nameif inside

security-level 100

ip address 192.168.1.1 255.255.255.0

!

interface GigabitEthernet1/3

nameif dmz

security-level 70

ip address 192.168.2.1 255.255.255.0

!

interface GigabitEthernet1/4

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/5

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/6

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/7

shutdown

no nameif

no security-level

no ip address

!

interface GigabitEthernet1/9

shutdown

no nameif

no security-level

no ip address

!

interface Management1/1

management-only

shutdown

no nameif

no security-level

no ip address

!

ftp mode passive

dns server-group DefaultDNS

domain-name ccnasecurity.com

object network inside-net

subnet 192.168.1.0 255.255.255.0

object network dmz-server

host 192.168.2.3

access-list OUTSIDE-DMZ extended permit ip any host 192.168.2.3

pager lines 24

mtu outside 1500

mtu inside 1500

mtu dmz 1500

icmp unreachable rate-limit 1 burst-size 1

no asdm history enable

arp timeout 14400

no arp permit-nonconnected

arp rate-limit 16384

!

object network inside-net

nat (inside,outside) dynamic interface

object network dmz-server

nat (dmz,outside) static 209.165.200.227

access-group OUTSIDE-DMZ in interface outside

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

timeout xlate 3:00:00

timeout pat-xlate 0:00:30

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 sctp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout sip-provisional-media 0:02:00 uauth 0:05:00 absolute

timeout tcp-proxy-reassembly 0:01:00

timeout floating-conn 0:00:00

timeout conn-holddown 0:00:15

timeout igp stale-route 0:01:10

user-identity default-domain LOCAL

aaa authentication telnet console LOCAL

aaa authentication ssh console LOCAL

aaa authentication http console LOCAL

aaa authentication login-history

http server enable

http 192.168.1.0 255.255.255.0 inside

no snmp-server location

no snmp-server contact

service sw-reset-button

crypto ipsec security-association pmtu-aging infinite

crypto ca trustpool policy

telnet 192.168.1.0 255.255.255.0 inside

telnet timeout 10

ssh stricthostkeycheck

ssh 192.168.1.0 255.255.255.0 inside

ssh timeout 10

ssh key-exchange group dh-group1-sha1

console timeout 0

threat-detection basic-threat

threat-detection statistics access-list

no threat-detection statistics tcp-intercept

webvpn

enable outside

cache

disable

error-recovery disable

group-policy SSL-VPN-POLICY internal

group-policy SSL-VPN-POLICY attributes

vpn-tunnel-protocol ssl-clientless

webvpn

url-list value Web_Server

dynamic-access-policy-record DfltAccessPolicy

username SSL-VPN-USER password $sha512$5000$N1ajK/bSMww9HuD/r6YLFA==$zOJ/ege6l65QAsHLQCgZfA== pbkdf2 privilege 0

username SSL-VPN-USER attributes

vpn-group-policy SSL-VPN-POLICY

username admin01 password $sha512$5000$oEhbf+nhDQXP07KnlaJ2jA==$DA6QQI066mXsAKo4erKlOg== pbkdf2

tunnel-group SSL-VPN type remote-access

tunnel-group SSL-VPN general-attributes

default-group-policy SSL-VPN-POLICY

!

class-map inspection_default

match default-inspection-traffic

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map global_policy

class inspection_default

inspect ftp

inspect h323 h225

inspect h323 ras

inspect ip-options

inspect netbios

inspect rsh

inspect rtsp

inspect skinny

inspect esmtp

inspect sqlnet

inspect sunrpc

inspect tftp

inspect sip

inspect xdmcp

inspect dns preset_dns_map

inspect icmp

policy-map type inspect dns migrated_dns_map_2

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map type inspect dns migrated_dns_map_1

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

!

service-policy global_policy global

prompt hostname context

no call-home reporting anonymous

call-home

profile CiscoTAC-1

no active

destination address http https://tools.cisco.com/its/service/oddce/services/DDCEService

destination address email callhome@cisco.com

destination transport-method http

subscribe-to-alert-group diagnostic

subscribe-to-alert-group environment

subscribe-to-alert-group inventory periodic monthly

subscribe-to-alert-group configuration periodic monthly

subscribe-to-alert-group telemetry periodic daily

Cryptochecksum:e875444854165549c9bb97a5a0116a75

: end

Router R1

R1#show run

Building configuration...

Current configuration : 2116 bytes

!

! Last configuration change at 21:04:09 UTC Tue Jan 6 2015

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$NxjzpK6dd08tuk$QDF7wpeiijoH80x.yVc2pwsODmSbabN5d0OQmOgHgEk

!

no aaa new-model

memory-size iomem 15

!

ip domain name ccnasecurity.com

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

!

voice-card 0

!

license udi pid CISCO2911/K9 sn FTX1713ALKC

license accept end user agreement

license boot module c2900 technology-package securityk9

license boot module c2900 technology-package uck9

license boot module c2900 technology-package datak9

!

username admin01 secret 9 $9$bv0p..7dLnvX0k$ho53wilFSM.lBCd.P97.PqbWhZMYqPMTDwBQ3lbcO8g

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 209.165.200.225 255.255.255.248

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 10.1.1.1 255.255.255.252

clock rate 125000

!

interface Serial0/0/1

no ip address

shutdown

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Serial0/0/0

!

control-plane

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

gatekeeper

shutdown

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1
line vty 0 4
 exec-timeout 5 0
 logging synchronous
 login local
 transport input ssh
!
scheduler allocate 20000 1000
!
end

Router R2

R2#show run

Building configuration...

Current configuration : 2138 bytes

!

! Last configuration change at 17:05:46 UTC Thu Nov 30 2017

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R2

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$F9BnTpKC4XoNcE$rMzDpGtwCFY24D6hdyppkKAUVE/DZjNiVG9Yj4OWhrc

!

no aaa new-model

memory-size iomem 15

!

ip domain name ccnasecurity.com

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

!

voice-card 0

!

license udi pid CISCO2911/K9 sn FTX1713ALJP

license boot module c2900 technology-package securityk9

license boot module c2900 technology-package uck9

license boot module c2900 technology-package datak9

!

username admin01 secret 9 $9$XeXYLYiVZL9FZ.$FzD3Tx4zhvG1QUsKAD1MxebR2sKn2IFj9gS4BJy5gFg

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 10.1.1.2 255.255.255.252

!

interface Serial0/0/1

ip address 10.2.2.2 255.255.255.252

clock rate 125000

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 172.16.3.0 255.255.255.0 Serial0/0/1

ip route 209.165.200.224 255.255.255.248 Serial0/0/0

!

control-plane

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

gatekeeper

shutdown

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 5 0

logging synchronous

login local

transport input ssh

!

scheduler allocate 20000 1000

!

end

Router R3

R3#show run

Building configuration...

Current configuration : 2149 bytes
!
! Last configuration change at 21:04:34 UTC Tue Jan 6 2015
!
version 15.4
service timestamps debug datetime msec
service timestamps log datetime msec
no service password-encryption
!
hostname R3
!
boot-start-marker
boot-end-marker
!
security passwords min-length 10
enable secret 9 $9$qGZYsq994ywdHk$N9cQxG82ViaHFjPzNn/mngPNdjK7MZhzRxcTcEvlWZw
!
no aaa new-model
memory-size iomem 15
!
ip domain name ccnasecurity.com
ip cef
no ipv6 cef
!
multilink bundle-name authenticated
!         
cts logging verbose
!
voice-card 0
!
license udi pid CISCO2911/K9 sn FTX1713ALJV
license accept end user agreement
license boot module c2900 technology-package securityk9
license boot module c2900 technology-package uck9
license boot module c2900 technology-package datak9
!         
vtp domain TSHOOT
vtp mode transparent
username admin01 secret 9 $9$ymTtFX0dTRK.eE$FnqHHtnGk0I8QXUSycOThp5bGj2wFREstIhuYcgRoJk
!
redundancy
!
interface Embedded-Service-Engine0/0
 no ip address
 shutdown
!
interface GigabitEthernet0/0
 no ip address
 shutdown
 duplex auto
 speed auto
!
interface GigabitEthernet0/1
 ip address 172.16.3.1 255.255.255.0
 duplex auto
 speed auto
!
interface GigabitEthernet0/2
 no ip address
 shutdown
 duplex auto
 speed auto
!
interface Serial0/0/0
 no ip address
 shutdown 
 clock rate 125000
!
interface Serial0/0/1
 ip address 10.2.2.1 255.255.255.252
!
ip forward-protocol nd
!
no ip http server
no ip http secure-server
!
ip route 0.0.0.0 0.0.0.0 Serial0/0/1
!
control-plane
!
mgcp behavior rsip-range tgcp-only
mgcp behavior comedia-role none
mgcp behavior comedia-check-media-src disable
mgcp behavior comedia-sdp-force disable
!
mgcp profile default
!
gatekeeper
 shutdown
!
line con 0
 exec-timeout 5 0
 logging synchronous
 login local
line aux 0
line 2
 no activation-character
 no exec
 transport preferred none
 transport output lat pad telnet rlogin lapb-ta mop udptn v120 ssh
 stopbits 1
line vty 0 4
 exec-timeout 5 0
 logging synchronous
 login local
 transport input none
!
scheduler allocate 20000 1000
!
end

Switches S1, S2 and S3 – Use default configs, except for host name